# introduction to the board and workspace setup

## goal

The goal of this lab is to introduce you to the Raspberry Pi 5 board and to provide instructions on how to set up the workspace that will be used for the following labs.

## pre-requisites

To follow this lab, you will need:

- Raspberry Pi 5 board full;

- USB-C power supply for Raspberry Pi 5 (official 27W Raspberry Pi 5 power supply recommended);

- 8 GB Micro SD card (32 GB card recommended);

- USB-to-Serial debug module for Raspberry Pi 5 or USB-to-TTL adapter;

- A host PC installed with Ubuntu Desktop 22.04 LTS or 24.04 LTS, or a virtual machine hosting Ubuntu Desktop with at least 100 GB of free space (recommend Ubuntu 24.04 LTS natively but it’s possible to run via Windows WSL 2);

- A Micro SD card reader attached to the PC/virtual machine;

- An Ethernet cable connecting the host PC/virtual machine with the Raspberry Pi 5;

- (Optional) Micro HDMI cable.

## The Raspberry Pi 5

The Raspberry Pi 5 is the successor to earlier versions of the Raspberry Pi single board computer. It features four 64-bit Arm cores and a wide range of connectivity including dual 4K HDMI output, dual-band 802.11ac WiFi, Bluetooth 5 and Bluetooth Low Energy, two USB 3 ports and two USB 2 ports, Gigabit Ethernet, MIPI camera/display transceivers, PCIe 2.0 and a standard GPIO style 40-pin header.

The Raspberry Pi 5 board measures 85mm x 56mm x 17mm, with a slight overlap for the SD card and connectors that extend over the edges. The SoC is a Broadcom BCM2712. This contains a quad-core Cortex-A76 running at 2.4 GHz and a VideoCore VII GPU.

|  | microSD slot (underneath) microSD slot (underneath) PCIe 2.0 interface PCIe 2.0 interface USB-C power socket (5V/5A) USB-C power socket (5V/5A) gigabit ethernet gigabit ethernet 2x USB2 (black), 2xUSB3 (blue) ports 2x USB2 (black), 2xUSB3 (blue) ports 2x micro-HDMI ports 2x micro-HDMI ports power button power button Broadcom BCM2712 SoC, 4x AArch64 cores Broadcom BCM2712 SoC, 4x AArch64 cores 40-pin GPIO header 40-pin GPIO header 2x MIPI camera/display transceivers 2x MIPI camera/display transceivers |
| --- | --- |
| Front View |  |

## workplace setup

The setup is divided into two parts: PC setup and board setup.

### PC setup

The following operations must be performed once to set up the Ubuntu Linux distribution you are using so that it is ready for building embedded Linux on the Raspberry Pi 5.

Install the following packages:

sudo apt install gawk wget git diffstat unzip texinfo gcc-multilib build-essential

sudo apt install chrpath socat libsdl1.2-dev xterm ncurses-dev lzop lz4

sudo apt install minicom u-boot-tools curl

You will need to install the latest version of Python. Type python3 -–version to check, or use the following command to download it.

sudo apt install python3

If you are using Git for the first time on your system, you should set your username and email address as follows:

git config --global user.name "my username"

git config --global user.email "myemail@example.com"

Download the embedded Linux source code for the Raspberry Pi 5, using the Scarthgap Long Term Support version of Yocto:

mkdir -p ~/raspberrypi5/sources

cd ~/raspberrypi5/sources

git clone -b scarthgap git://git.yoctoproject.org/poky

git clone -b scarthgap git://git.openembedded.org/meta-openembedded

git clone -b scarthgap https://github.com/agherzan/meta-raspberrypi

Note: Be patient. These operations may take several minutes as around 400 megabytes of source code files need to be downloaded.

It is now time to define a specific network configuration for the lab experiments. For this purpose, create the following directory:

cd ~/raspberrypi5/sources/poky/meta/recipes-core/init-ifupdown/init-ifupdown-1.0/

mkdir raspberrypi5

In this directory, place a file named “interfaces” with the following content:

# /etc/network/interfaces -- configuration file for ifup(8), ifdown(8)

# The loopback interface

auto lo

iface lo inet loopback

# Wireless interfaces

iface wlan0 inet dhcp

wireless_mode managed

wireless_essid any

wpa-driver wext

wpa-conf /etc/wpa_supplicant.conf

# that’s a lower case letter L in atml0

iface atml0 inet dhcp

# Wired or wireless interfaces

auto eth0

iface eth0 inet static

address 192.168.1.2

netmask 255.255.255.0

gateway 192.168.1.1

network 192.168.1.0

# Bluetooth networking

iface bnep0 inet dhcp

You can copy and adapt the interfaces file from the parent directory. Among the other activities, this file sets the IP address for the Ethernet device to the fixed address 192.168.1.2.

Configure the environment to build the source just downloaded for the Raspberry Pi 5, and accept the license agreement:

cd ~/raspberrypi5/

source sources/poky/oe-init-build-env rpi-build

This generates a fresh build environment, and moves to a new directory called `rpi-build`

Using your preferred text editor program (for example, vim, emacs, nano, gedit) edit the file “rpi-build/conf/local.conf”, remove the existing content, and type or paste in the following code to include the needed information for building embedded Linux for the Raspberry Pi 5:

MACHINE ?= "raspberrypi5"

PREFERRED_VERSION_linux-raspberrypi = "6.6%"

DISTRO_FEATURES:remove = "x11 wayland"

ENABLE_UART = "1"

BOOT_SPACE = "51200"

LICENSE_FLAGS_ACCEPTED = "synaptics-killswitch"

IMAGE_FSTYPES:append = " rpi-sdimg"

It may be necessary to change your PREFERRED_VERSION_linux-raspberrypi variable. This is the kernel version to be built for your image. The ‘%’ character is a wildcard that matches the most recent supported version in Yocto.

Using your preferred text editor, edit the file “rpi-build/conf/bblayers.conf”, remove the existing content, and type or paste in the following code to include the necessary information for building embedded Linux for the Raspberry Pi 5. Ensure it is formatted exactly as shown, because spaces and missing indents can cause parsing errors.

LCONF_VERSION = "7"

BBPATH = "${TOPDIR}"

BSPDIR := "${@os.path.abspath(os.path.dirname(d.getVar('FILE', True)) + '/../..')}"

BBFILES ?= ""

BBLAYERS ?= " \

${BSPDIR}/sources/poky/meta \

${BSPDIR}/sources/poky/meta-poky \

${BSPDIR}/sources/poky/meta-yocto-bsp \

${BSPDIR}/sources/meta-openembedded/meta-oe \

${BSPDIR}/sources/meta-openembedded/meta-python \

${BSPDIR}/sources/meta-openembedded/meta-multimedia \

${BSPDIR}/sources/meta-raspberrypi \

"

Once this operation is completed, the environment is ready for building embedded Linux for the Raspberry Pi 5.

Ensure that you are still in the build directory “rpi-build”. Linux for the Raspberry Pi 5 can now be built by issuing the following command:

bitbake rpi-test-image

Note: Be patient. This operation may take up to two hours, depending on the PC you are using, as many megabytes of source code must be compiled. If you are compiling the image on a virtual machine, it may take several hours.

Note: If the `bitbake` execution reports an ERROR, either a PermissionError or a User namespaces error, you need to make a temporary change to your Ubuntu security profile to enable `bitbake` to run:

sudo su

echo 0 > /proc/sys/kernel/apparmor_restrict_unprivileged_userns

exit

See https://lists.yoctoproject.org/g/yocto/topic/workaround_for_uid_map_error/106192359 for more details

Note: If the `bitbake` execution reports an ERROR at the stage when it is generating the SD card image (do_img_rpi_sdimg), it may be necessary to increase the SD card image size. In the file “rpi-build/conf/local.conf”, add the following variable definition to increase the boot sector size to 50MB:

BOOT_SPACE = "51200"

Once the build is completed, the SD card image for booting the Raspberry Pi 5 can be found at the following path:

raspberrypi5/rpi-build/tmp-glibc/deploy/images/raspberrypi5/rpi-test-image-raspberrypi5.rootfs.rpi-sdimg

The image can be copied to a Micro SD card using appropriate Linux tools (assuming the Micro SD card is available to the PC as /dev/sdb). Alternatively, and more conveniently, a flash program could be used.

First, run the

sudo fdisk -l

command to determine which device to flash to (plug in and unplug the SD card to identify it). In this example, the SD card is named “sdb” (this may differ in your environment). Next, ensure that the device is unmounted. This can be done using the following command:

sudo umount /dev/sdb*

Once this is done, use the following command to copy the image to the SD card (substitute any folder names and device names to match your specific environment).

sudo dd bs=1M if=/home/user/raspberrypi5/rpi-build/tmp-glibc/deploy/images/raspberryp5/rpi-test-image-raspberrypi5.rootfs.rpi-sdimg of=/dev/sdc

This may take a minute or so to transfer the data to the SD card. Note that if this is not done correctly, the image flashed to the SD card may cause problems when attempting to power on the board. In such a case, retry the process and ensure that it is completed properly, or use a flash program to automate it.

While the SD card is still connected to the development host PC, use the following commands to navigate to the etc folder on the SD card (assuming the device has been mounted – in Ubuntu, you can click on the removable storage icon in the file explorer to mount the device).

cd /media/user/SD_name/etc – Use the ls command to find the name of the SD card in the user folder.

Then, use a terminal text editor to open the shadow file.

sudo vim shadow

Alternatively, you can use the following command to update the shadow file

sudo mkdir -p /mnt/sdroot

sudo mount /dev/sda2 /mnt/sdroot

sudo vim /mnt/sdroot/etc/shadow

Check that there are no characters between the first two colons in the first line. If there is (for example, an asterisk), then remove it so that the first line appears as follows (perhaps with different numbers):

root::17728:0:99999:7:::

Exit the text editor vi by pressing Esc, then typing `:wq!`.

Now unmount the SD card and remove it from the SD card reader.

Next, connect to the board and boot it. You can connect via a Serial TTY using Minicom, over Ethernet, or directly interact with the system by connecting it to an HDMI monitor and a USB keyboard.

### Minicom setup

Connect the USB-to-Serial debug module for the Raspberry Pi 5 or a USB-to-TTL adapter to J8 on the Raspberry Pi 5 board (pins 8 and 10 are UART0_TX and UART0_RX, which are the lines where the serial console of BCM2712 is routed). Then, configure the communication parameter to 115200 8N1 in your preferred serial terminal emulation program (under Linux, use the device /dev/ttyUSB0 or /dev/ttyUSB1).

Once the programming of the Micro SD card is complete, you are ready to configure the communication program used to connect to the board: Minicom. Issue the following command to start Minicom in setup mode:

sudo minicom -s

Then, perform the following steps:

- Select “Serial port setup” in the configuration menu;

- Press A to modify the Serial Device and to set it to “/dev/ttyUSB0”;

- Press F to set Hardware Flow Control to “No”;

- Press Enter to return to the previous menu;

- Select “Save setup as dfl”, to save the setup as default configuration;

- Select “Exit” from Minicom.

Optional: The Raspberry Pi 5 Ethernet interface is configured to acquire an IP address via DHCP. If you intend to connect the board directly to your PC, you must configure the Ethernet connection properly. It is recommended to create a new Ethernet connection on your PC using the following statically assigned IPv4 configuration:

IP Address: 192.168.1.1
Netmask: 255.255.255.0
Gateway: 192.168.1.1

### Board setup

- Insert the newly written Micro SD card into the SD slot of the Raspberry Pi 5;

- (Optional) connect the Micro HDMI cable to the Raspberry Pi 5 connector;

- (Optional) connect the USB-to-Serial debug module between the Pi and the host device

- Connect the power supply to the Raspberry Pi 5;

- Also, connect the board to your host device using an Ethernet cable.

Once the power cable is connected, the board will power on. If you connected the Micro HDMI cable, a splash screen will appear on the monitor, along with a progression bar indicating the bootstrap is running.

After the bootstrap is completed, a new device will appear on the PC: /dev/ttyUSB0. This device is the virtual serial port through which you can access the Raspberry Pi 5 by issuing the following command:

sudo minicom

If all previous steps have been completed successfully, the following message will appear on the terminal:

Welcome to minicom 2.7

OPTIONS: I18n

Compiled on Jan  1 2014, 17:13:19.

Port /dev/ttyUSB0, 10:22:24

Press CTRL-A Z for help on special keys

[    0.000000] Booting Linux on physical CPU 0x0

[    0.000000] Initializing cgroup subsys cpuset

[    0.000000] Initializing cgroup subsys cpu

[    0.000000] Initializing cgroup subsys cpuacct

......

OpenEmbedded nodistro.0 raspberrypi5 ttyS0

raspberrypi5 login:

You can now login as “root” (no password required).

Note:

- To open the Minicom menu, press Ctrl + A and then Z (either lowercase or uppercase).

Optional: (This step is already complete if you performed the config step above.) The Raspberry Pi 5 Ethernet interface is configured by default to acquire an IP address via DHCP. If you intend to connect the board directly to your PC, you must configure the Ethernet connection properly. It is recommended to create a new Ethernet connection on the Raspberry Pi 5 by editing the file /etc/network/interfaces and modifying the configuration of the network device eth0 as follows:

# Wired or wireless interfaces

auto eth0

iface eth0 inet static

address 192.168.1.2

netmask 255.255.255.0

gateway 192.168.1.255

Test whether the network is configured properly by using the following command:

ifconfig

You should see an address of 192.168.1.2. However, if this is not the case, you can run the following commands:

ifdown eth0

ifup eth0

After running these two commands, the network should be configured properly.

If you are using a Linux virtual machine as the host for the sole purpose of communication between the board and the host, you may need to switch the “Network” settings to a “Bridged Adapter” and select the Ethernet device being used.

Once the PC and the Raspberry Pi 5 are configured, you can test the connection using the following command on the Pi:

ping 192.168.1.1

If the configuration is correct, a message similar to the following will appear on the terminal:

root@raspberrypi5:~# ping 192.168.1.1

PING 192.168.1.1 (192.168.1.1) 56(84) bytes of data.

64 bytes from 192.168.1.1: icmp_seq=1 ttl=64 time=1.54 ms

64 bytes from 192.168.1.1: icmp_seq=2 ttl=64 time=1.61 ms

64 bytes from 192.168.1.1: icmp_seq=3 ttl=64 time=1.98 ms

64 bytes from 192.168.1.1: icmp_seq=4 ttl=64 time=1.73 ms

# SUMMARY

In this session, we built an initial version of Linux image with Yocto Project. One of the main benefits of using Yocto Project is that the Linux image can be flexibly customized to accommodate specific use cases. The key to configuring the image is the inclusion of different layers and recipes.

There are two files that determine the above features, conf/local.conf and conf/bblayers.conf. To gain a better understanding of these two files and the customization feature of Yocto Project, we shall carry out additional practice in future labs.