Quiz Questions – Embedded Linux

Lecture 5:

Question 1 – “What components are required for interfacing with custom IO devices?”

- A root file system and a device driver

- A kernel space application and a device driver

- A user space application and a device driver

- An inode and a device driver

Question 2 – “Which of the following are methods that the CPU can use to recognize that an IO has data ready to be read?”

- Interrupt

- Request

- Observe

- Polling

Question 3 – “Which of the following are transfer modes of ‘direct memory access’?”

- Synchronous

- Burst

- Transparent

- Cycle stealing

Question 4 – “Which one of the following is not a Linux recognized device class?”

- Character devices

- Block devices

- Input devices

- Network interfaces

Question 5 – “What is a device file?”

- An intermediator through which a kernel application can exchange data with a device driver

- An intermediator through which a user application can exchange data with a device driver

- An intermediator through which a user application can exchange data with the kernel

- A file that hosts the information regarding a specific device