# READING TEMPERATURE sensor using linux kernel module WITH AI INTEGRATION

## goal

The goal of this lab is to build a loadable kernel module that provides access to the Raspberry Pi’s on-board temperature sensor and then integrate the temperature data with llama.cpp to generate AI-based status reports from a local large language model (LLM). The entire system will be constructed using the Yocto build environment.

## prerequisites

To follow this Lab, you need:

- Raspberry Pi 5 board full;

- USB-C power supply for Raspberry Pi 5 (official 27W Raspberry Pi 5 power supply recommended);

- 8 GB Micro SD card (32 GB card recommended);

- USB-to-Serial debug module for Raspberry Pi 5 or USB-to-TTL adapter;

- A host PC installed with Ubuntu Desktop 22.04 LTS or 24.04 LTS, or a virtual machine hosting Ubuntu Desktop with at least 100 GB of free space (recommend Ubuntu 24.04 LTS natively, but it’s possible to run via Windows WSL 2);

- A Micro SD card reader attached to the host PC/virtual machine;

- An Ethernet cable connecting the host PC/virtual machine with the Raspberry Pi 5;

- (Optional) Micro HDMI cable.

## Introduction

The Raspberry Pi 5 has an on-board CPU temperature sensor that can be queried using Linux kernel code.

In this lab, a loadable kernel module is implemented that reads the temperature sensor and reports this data via a sysfs kernel object.

The temperature sensor data can then be integrated with a locally deployed large language model (LLM) to provide AI-generated, real-time text based on the live temperature sensor values. If you prefer, you can use a different sensor. This lab uses the on-board temperature sensor as no extra hardware is required.

## workplace setup

Assuming you have completed the previous lab, on your Ubuntu host PC, change to the raspberrypi5 directory and prepare the build environment:

cd ~/raspberrypi5/

source sources/poky/oe-init-build-env rpi-build

You now have the system ready to build the embedded Linux distribution for the Raspberry Pi 5 board.

## Preparing the recipes

In this lab, we are adding a new device driver to be compiled as a loadable kernel module, as you may need to do if you are asked to customize the board support package for the Raspberry Pi 5 to support a new I/O device attached to the system.

For this purpose, we will add a new Yocto recipe to the meta-raspberrypi layer. To support this, create the following directory:

cd ~/raspberrypi5/sources/meta-raspberrypi/recipes-kernel

mkdir -p temp-mod/files

In temp-mod,put a file named temp_1.0.bb with the following content:

| 0: 1: 2: 3: 4: 5: 6: 7: 8: 9: | DESCRIPTION = "temperature driver" LICENSE = "GPL-2.0-or-later" LIC_FILES_CHKSUM = "file://${BPN}.c;md5=f446e89143ba482392ed18f09755f170" inherit module PR = "r0" SRC_URI = "file://Makefile file://${BPN}.c" S = "${WORKDIR}" |
| --- | --- |

This file describes the property of the recipe, and in particular:

- Line 0 gives a simple description of the recipe’s purpose;

- Lines 1-2 reference the license under which the recipe is distributed and specify the license file and its checksum (ideally the license text should be included in the .c file);

- Line 4 indicates that the recipe inherits the properties required to build kernel modules;

- Line 6 specifies the machine layer for which the recipe is intended;

- Line 8 lists the source files needed to build the recipe;

- Line 9 sets the symbol that points to the working directory used to perform the build operations.

In the temp-mod/files directory, create a file named temp.c with the following content:

|  | #include <linux/module.h> #include <linux/kernel.h> #include <linux/init.h> #include <linux/kobject.h> #include <linux/sysfs.h> #include <linux/thermal.h> static struct kobject *temp_kobj; static ssize_t temp_show(struct kobject *kobj, struct kobj_attribute *attr, char *buf) { struct thermal_zone_device *tz; int temp_millic, ret; tz = thermal_zone_get_zone_by_name("cpu-thermal"); if (IS_ERR(tz)) return sysfs_emit(buf, "Error: cannot find default thermal zone\n"); ret = thermal_zone_get_temp(tz, &temp_millic); if (ret) return sysfs_emit(buf, "Error: cannot read temp\n"); return sysfs_emit(buf, "%d\n", temp_millic / 1000); } static struct kobj_attribute temp_attribute = __ATTR(value, 0444, temp_show, NULL); static int __init temp_init(void) { int error = 0; temp_kobj = kobject_create_and_add("temp", kernel_kobj); if (!temp_kobj) return -ENOMEM; error = sysfs_create_file(temp_kobj, &temp_attribute.attr); if (error) { pr_err("temp: failed to create sysfs file\n"); kobject_put(temp_kobj); } pr_info("temp module loaded\n"); return error; } static void __exit temp_exit(void) { kobject_put(temp_kobj); pr_info("temp module unloaded\n"); } module_init(temp_init); module_exit(temp_exit); MODULE_LICENSE("GPL"); MODULE_AUTHOR("Jeremy Singer"); MODULE_DESCRIPTION("Kernel module exposing integer Celsius temp via sysfs"); |
| --- | --- |

Finally, in the same directory, create a file named Makefile with the following content:

| 0: 1: 2: 3: 4: 5: 6: 7: 8: 9: 10: 11: 12: 13: 14: | obj-m := temp.o SRC := $(shell pwd) all: $(MAKE) -C $(KERNEL_SRC) M=$(SRC) modules_install: $(MAKE) INSTALL_MOD_DIR=kernel/drivers/temp_mod -C $(KERNEL_SRC) M=$(SRC) modules_install clean: rm -f *.o *~ core .depend .*.cmd *.ko *.mod.c rm -f Module.markers Module.symvers modules.order rm -rf .tmp_versions Modules.symvers |
| --- | --- |

The file describes how the source code should be built and where the output should be placed, specifically:

- Lines 0-5 define the object module to be created, locate the current directory, and invoke the build command, which refers to the source and kernel source symbols maintained by Yocto;

- Lines 7-8 define the operations required to install the loadable kernel module resulting from the compilation. In this example, the object will be installed in the root file system in the root file system under the /lib/modules/<kernel version>/kernel/drivers/temp_mod directory.

- Lines 10-14 define the operations performed when cleaning the build outputs (i.e., removing the temporary files and the compilation outputs).

Once these operations are completed, you must update the machine layer configuration to include the new driver. To do this, edit the file:

~/raspberrypi5/rpi-build/conf/local.conf

Add the following statement as the last line of the file:

IMAGE_INSTALL:append = " temp"

This line tells Yocto that, when building Linux, the newly created device driver should be built and included in the root file system.

## Building and deploying the new system

You are now ready to build the new system as follows:

source ~/raspberrypi5/sources/poky/oe-init-build-env rpi-build

bitbake -c clean rpi-test-image

bitbake rpi-test-image

During the build, the compiler may detect a discrepancy in the license reference. Copy and paste the suggested reference (“the new md5 checksum is..”) into your temp_1.0.bb file, and then start the build again.

After a while, a new Micro SD card image will be generated, which you can deploy in the Micro SD card as follows (assuming the card is available to the PC as /dev/sdN). Alternatively, you can use a program of your choice to flash the image.

First, run the command:

sudo fdisk -l

This helps you to determine which device to flash to (plug in and unplug the SD card to identify it). In this example, the SD card is under the name “sdb” (this may differ in your environment). Next, ensure that the device is unmounted. This can be done using the following command:

sudo umount /dev/sdb*

Once this is done, use the following command to copy the image to the SD card (substitute any folder names and device names to match your specific environment).

sudo dd bs=1M if=/home/user/raspberrypi5/rpi-build/tmp-glibc/deploy/images/raspberrypi5/rpi-basic-image-raspberrypi5.rootfs.rpi-sdimg of=/dev/sdb

Note that if this is not done correctly, the image flashed to the SD card may cause problems when attempting to power on the board. If this happens, retry the process and ensure that it is completed properly, or use a flashing program to automate the process.

While the SD card is still connected to the development host, use the following commands to navigate to the etc folder on the SD card (assuming the device has been mounted).

cd media/user/SD_name/etc  # Use the ls command to find the name of the SD card in the user folder.

Then, use a terminal text editor to open the shadow file.

sudo vim shadow

or

sudo gedit shadow

Check that there are no characters between the first two colons in the first line. If there are any characters, remove them so that the first line appears as follows:

root::17728:0:99999:7:::

This allows root login without a password. Now save the file and exit the text editor.

## Running the module

After booting the new Linux system, you can verify whether the build process was completed successfully. After logging into the Raspberry Pi 5 as the root user (no password), enter the following commands:

root@raspberrypi5:/# ls -la /lib/modules/$(uname-r)/kernel/drivers/temp_mod

drwxr-xr-x    2 root     root          1024 May 15 08:47 .

drwxr-xr-x   43 root     root          1024 May 10 12:09 ..

-rw-r--r--    1 root     root          9080 May 12 03:02 gpio.ko.xz

-rw-r--r--    1 root     root         10140 May 15 08:22 temp.ko.xz

-rw-r--r--    1 root     root          5384 May 11 08:05 hello.ko.xz

root@raspberrypi5:/#

The directory in the root file system contains temp.ko.xz, which is the compressed kernel object that includes the binary code for the loadable kernel module.

You can now insert the module into the kernel as follows:

root@raspberrypi5:/# insmod /lib/modules/ $(uname -r/kernel/drivers/my_mod/temp.ko.xz

[  143.363069] Loading temp_module

The module should be loaded correctly; you can check this via the dmesg command.

To test the module, you can read data from the installed sysfs file:

root@raspberrypi5:/# cat /sys/kernel/temp/value

39

This displays the temperature sensor value in degrees Celsius.

In this example, the virtual file system API is used to trigger the sensor reading, and sysfs is used to read the measured value.

To remove the module, do the following:

root@raspberrypi5:/# rmmod temp

After removing the module, you can verify that the temp directory is no longer available in /sys/kernel.

## ADDING AN AI MODEL

The next step is to integrate this sensor module with AI. We will use a local model so that it can be deployed without requiring constant access to the cloud. A local model also helps avoid accumulating expensive API costs.

First, we will add a custom layer to our Yocto build as follows:

cd ~/raspberrypi5/sources/

mkdir -p meta-llama/recipes-llm/llama-cpp/

cd meta-llama/recipes-llm/llama-cpp/

In this newly created folder, add the following content to a new bitbake recipe file called llama-cpp.bb

| 0: 1: 2: 3: 4: 5: 6: 7: 8: 9: 10: 11: 12: 13: 14: 15: 16: 17: 18: 19: 20: 21: 22: 23: 24: 25: 26: 27: 28: 29: 30: 31: 32: 33: 34: 35: 36: 37: 38: 39: 40: | SUMMARY = "llama.cpp - LLM inference engine for local models" DESCRIPTION = "llama.cpp is a C++ implementation of inference for LLaMA and other models in GGUF format" HOMEPAGE = "https://github.com/ggml-org/llama.cpp" LICENSE = "MIT" LIC_FILES_CHKSUM = "file://LICENSE;md5=3b83ef96387f14655fc854ddc3c6bd57" SRCREV = "360d6533db39e11577afe9b0aece20c6b5ddaf1f" SRC_URI = " \ git://github.com/ggml-org/llama.cpp.git;branch=master;protocol=https \ " S = "${WORKDIR}/git" DEPENDS = "cmake" inherit cmake FILES:${PN} += "${bindir}/llama-cli" FILES:${PN} += " \ ${libdir}/libggml-base.so \ ${libdir}/libggml-cpu.so \ ${libdir}/libggml.so \ ${libdir}/libmtmd.so \ ${libdir}/libllama.so \ ${libdir}/libggml-cpu.so \ " INSANE_SKIP:${PN} += "buildpaths already-stripped dev-deps" INSANE_SKIP:${PN}-dev += "dev-elf" do_install() { install -d ${D}${bindir} install -d ${D}${libdir} install -m 0755 ${B}/bin/llama-cli ${D}${bindir}/llama-cli install -m 0644 ${B}/bin/libllama.so ${D}${libdir}/libllama.so install -m 0644 ${B}/bin/libggml.so ${D}${libdir}/libggml.so install -m 0644 ${B}/bin/libggml-base.so ${D}${libdir}/libggml-base.so install -m 0644 ${B}/bin/libggml-cpu.so ${D}${libdir}/libggml-cpu.so } |
| --- | --- |

This recipe specifies how to build llama.cpp from its Git repository source.

We also need to add a configuration file for the meta-llama layer. In the sources/meta-llama/conf directory, add the following content to a file named layer.conf:

| 0: 1: 2: 3: 4: 5: 6: 7: 8: 9: 10: | # We have a conf and classes directory, add to BBPATH BBPATH .= ":${LAYERDIR}" # We have recipes-* directories, add to BBFILES BBFILES += "${LAYERDIR}/recipes-*/*/*.bb \ ${LAYERDIR}/recipes-*/*/*.bbappend" BBFILE_COLLECTIONS += "llama" BBFILE_PATTERN_llama = "^${LAYERDIR}/" BBFILE_PRIORITY_llama = "5" LAYERVERSION_llama = "4" LAYERSERIES_COMPAT_llama = "scarthgap" |
| --- | --- |

Now we can add this layer to our configuration by running the following command:

cd ~/raspberrypi5/rpi-build

bitbake-layers add-layer ../sources/meta-llama

You can check the inserted layer with the following command:

cd ~/raspberrypi5/rpi-build

bitbake-layers show-layers

Check that this component can be built properly using the following command:

cd ~/raspberrypi5/rpi-build

bitbake llama-cpp

This provides an LLM inference engine, but we still need to fetch a pre-trained model for llama.cpp. We can add a recipe to do this as well.

Again, we will add a custom recipe to the meta-llama layer as follows:

cd ~/raspberrypi5/sources/meta-llama/recipes-llm

mkdir gguf-model

cd gguf-model

In this newly created folder, add the following content to a new BitBake recipe file called gguf-model.bb.

| 0: 1: 2: 3: 4: 5: 6: 7: 8: 9: 10: 11: 12: 13: 14: | DESCRIPTION = "TinyLlama GGUF model file" LICENSE = "CLOSED" SRC_URI = "https://huggingface.co/TheBloke/TinyLlama-1.1B-Chat-v1.0-GGUF/resolve/main/tinyllama-1.1b-chat-v1.0.Q6_K.gguf" SRC_URI[sha256sum] = "9fb2be7c8bbc0ca358ae88cef6d9e6844b81929eaffa5016a543e6f56b22cb6d" FILES:${PN} = "/opt/llama-models/tinyllama-1.1b-chat-v1.0.Q6_K.gguf" S = "${WORKDIR}" do_install() { install -d ${D}/opt/llama-models install -m 0644 ${S}/tinyllama-1.1b-chat-v1.0.Q6_K.gguf ${D}/opt/llama-models/ } |
| --- | --- |

This recipe fetches a binary model from a standard online repository called Hugging Face. Note that these models can be large. We need one that fits on our limited device, both in terms of RAM and persistent storage. The selected model is around 1 GB in size.

Again, we can check this recipe is working properly by building it:

cd ~/raspberrypi5/rpi-build

bitbake gguf-model

Then, add these new components to the Raspberry Pi image by modifying the local configuration file rpi-build/conf/local.conf to include the following two lines at the end:

IMAGE_INSTALL:append = " temp llama-cpp gguf-model"

KERNEL_MODULE_AUTOLOAD:append = " temp"

This configuration file incorporates all three components we have just built: the temperature sensor kernel module, the LLM inference engine, and the pre-trained model.

Now, build the Raspberry Pi image:

cd ~/raspberrypi5/rpi-build

bitbake -c clean rpi-test-image

bitbake rpi-test-image

As before, this new SD card image needs to be written to the MicroSD card, and the /etc/shadow password for the root user must be removed.

Once the SD card is ready, you can boot the Raspberry Pi 5 and test it. After logging in to obtain a root shell, you should see that the temp kernel module is automatically loaded, as you can query the /sys/kernel/temp/value sysfs file.

Next, you can write a custom shell script with a query to the sysfs temperature file incorporated into an appropriate LLM prompt. Check the wide range of options for the llama-cli command using llama-cli --help. This shell script can be installed in /usr/bin, where the llama-cli executable is also located.

#!/bin/sh

TEMP=`cat /sys/kernel/temp/value`

llama-cli -m /opt/llama-models/tinyllama-1.1b-chat-v1.0.Q6_K.gguf \

-p "My CPU temperature is $TEMP degrees Celsius. What do you make of that?" \

-n 48 -no-cnv 2>/dev/null

## SUMMARY

In this lab, you installed a custom kernel module that exposed an internal temperature sensor reading. You then used Yocto custom layers to deploy a local LLM on the Raspberry Pi 5. Finally, you wrote a simple script to process the temperature sensor readings and generate AI-based textual responses. Can you imagine less contrived situations where this kind of AI processing pipeline might be useful?