# introduction to code development and debugging using yocto

## goal

The goal of this lab is to enable you to set up a development environment using Yocto to write and cross-compile applications for the embedded Linux running on the Raspberry Pi 5, and to perform remote debugging operations.

## prerequisites

To follow this Lab, you need:

- Raspberry Pi 5 board full;

- USB-C power supply for Raspberry Pi 5 (official 27W Raspberry Pi 5 power supply recommended);

- 8 GB Micro SD card (32 GB card recommended);

- USB-to-Serial debug module for Raspberry Pi 5 or USB-to-TTL adapter;

- A host PC installed with Ubuntu Desktop 22.04 LTS or 24.04 LTS, or a virtual machine hosting Ubuntu Desktop with at least 100 GB of free space (recommend Ubuntu 24.04 LTS natively.

- A Micro SD card reader attached to the host PC/virtual machine;

- An Ethernet cable connecting the host PC/virtual machine with the Raspberry Pi 5;

- (Optional) Micro HDMI cable.

## workplace setup

We assume that:

- The host PC or virtual machine is configured with the IP address 192.168.1.1;

- The Raspberry Pi 5 is configured with the IP address 192.168.1.2;

- The PC or virtual machine and the Raspberry Pi 5 are connected using an Ethernet cable.

Please refer to the first lab for instructions on how to obtain the above configuration.

## Preparing the recipes

To debug applications running on the Raspberry Pi 5, a new package must be added to the Yocto configuration so that the debugging tools are installed in the embedded Linux distribution. For this purpose, edit the file raspberryPi5 /rpi-build/conf/local.conf to include the following content:

MACHINE ?= "raspberrypi5"

DISTRO ?= "poky"

BOOT_SPACE = "51200"

PREFERRED_VERSION_linux-raspberrypi = "6.6.%"

DISTRO_FEATURES:remove = "x11 wayland"

EXTRA_IMAGE_FEATURES = "debug-tweaks tools-debug"

ENABLE_UART = "1"

LICENSE_FLAGS_ACCEPTED = "synaptics-killswitch"

IMAGE_FSTYPES:append = " rpi-sdimg"

The new configuration adds the tools-debug package, the tools-debug feature installs common debugging and profiling utilities such as gbd,gdbserver,perf,ftrace/trace-cmd,and strace on the targert.

## Building and deploying the new system

Move to the raspberrypi5 directory and prepare the build environment:

cd ~/raspberrypi5

source sources/poky/oe-init-build-env rpi-build

bitbake -c clean rpi-test-image

bitbake rpi-test-image

After a while, a new image will be built, which you can deploy to the Micro SD as follows (assuming the card is available to the PC as /dev/sdN). Alternatively, you can use a program of your choice to flash the image.

First, run the:

sudo fdisk -l

command to determine which device to flash (plug in and unplug the SD card to identify it). In this example, the SD card is named “sdb” (Caveat: this may be different in your environment, please make sure to flash on the correct device.  Otherwise, it might hurt your host machine.). Next, ensure that the device is unmounted. This can be done using the following command:

sudo umount /dev/sdb*

Once this is done, use the following command to copy the image to the SD card. Substitute any folder names and device names to match your specific environment.

sudo dd bs=1M if=/home/user/raspberrypi5/rpi-build/tmp/deploy/images/raspberrypi5/rpi-test-image-raspberrypi5.rpi-sdimg of=/dev/sdb

sync

Note that if this is not done properly, the image flashed to the SD card may cause problems when attempting to power on the board. If this happens, retry the process and ensure that it is completed properly, or use a flashing program to automate the process.

Also note that this time, the most recently built image should be in the “tmp” folder, not “tmp-glibc”.

## Building and deploying the new system

You are now ready to build yocto SDK which can be distributed for cross compiling, which allows you to build programs compatible with the embedded Linux system you have just compiled.

If you have restarted since setting up the environment, re-run the following setup commands:

cd ~/raspberrypi5

source sources/poky/oe-init-build-env rpi-build

To build the cross-compiler environment, execute the following command:

bitbake rpi-test-image –c populate_sdk

This command will create the file ~/raspberrypi5/rpi-build/tmp/deploy/sdk/poky-glibc-x86_64-rpi-test-image-cortexa76-raspberrypi5-toolchain-5.0.11.sh, which includes the binary files, libraries, and header files needed to build programs for the previously created embedded Linux distribution.

Note: The name of the cross-compiler file may vary. To find the correct file name, run the following command:

ls ~/raspberrypi5/rpi-build/temp/deploy/sdk

Look for the file with the .sh extension.

The cross-compiler environment should be installed on the development host PC as follows:

cd ~/raspberrypi5/rpi-build/tmp/deploy/sdk/

./poky-glibc-x86_64-rpi-basic-image-cortexa7hf-neon-vfpv4-toolchain-2.1.3.sh

The command will produce the following output:

Poky (Yocto Project Reference Distro) SDK installer version 5.0.11 =================================================================

Enter target directory for SDK (default: /opt/poky/5.0.11):  (press Enter)

You are about to install the SDK to "/opt/poky/5.0.11". Proceed[Y/n]?  (press Y Enter)

[sudo] password for user (type password and press Enter)

Extracting SDK................................................................done

Setting it up...done

SDK has been successfully set up and is ready to be used.

Each time you wish to use the SDK in a new shell session, you need to source the environment setup script e.g.

$ . /opt/poky/5.0.11/environment-setup-cortexa76-poky-linux

The cross-compiler environment is installed in /opt/poky/5.0.11/, and a shell script is provided to configure the variables needed to use it.

Before using the cross-compiler environment, the necessary environment variables must be set by running the following command:

. /opt/poky/5.0.11/environment-setup-cortexa76-poky-linux

## writing a simple test application and DEPLOYING it

Create a new directory and populate it with a simple “Hello, World!” application stored in the file hello.c, containing the following content:

#include <stdio.h>

int     main( void )

{

printf( "Hello, World!\n" );

return 0;

}

Now, prepare the file Makefile with the following content (as before, ensure the idented lines use tabs, not spaces):

CC=aarch64-poky-linux-gcc --sysroot=/opt/poky/5.0.11/sysroots/cortexa76-poky-linux/

LD=aarch64-poky-linux-ld --sysroot=/opt/poky/5.0.11/sysroots/cortexa76-poky-linux/

CFLAGS=-O2 -pipe -g -feliminate-unused-debug-types

CXXFLAGS=-O2 -pipe -g -feliminate-unused-debug-types

all: hello

hello.o: hello.c

hello: hello.o

$(CC) $(CFLAGS) -o hello hello.o

clean:

rm hello hello.o

Note: Update the Poky(Yocto Project) version accordingly.

This file specifies how to run the cross-compiler (arm-poky-linux-gcc) to generate the executable file to be run on the Raspberry Pi 5.

To build the program, run the following command:

make

If the cross-compilation is executed correctly, the file hello will be created. You can verify that it is an appropriate executable for the Raspberry Pi 5 target using the following command:

file hello

This should report that hello is an ELF executable for the Arm AArch64 architecture.

The executable program can be transferred from the host PC to the Raspberry Pi 5 using the scp command as follows:

scp hello root@192.168.1.2:~/hello

If the operation succeeds, you will find the hello executable in the /home/root directory on the Raspberry Pi 5.

It might display a warning. If this happens, you can remove the stale host key as suggested in the message.

ssh-keygen -f "/home/user/.ssh/known_hosts" -R 192.168.1.2

Then, run the scp command again, and it should copy the executable successfully.

An alternative transfer option is to mount the Micro SD card on your host PC, and to copy the hello file directly to the root file system using the standard cp command.

## Debugging the test application

You are now ready to remotely debug the application using the command-line debugger included with the cross-compilation environment.

On the Raspberry Pi 5, run the following command:

root@raspberrypi5:~# gdbserver localhost:2000 hello

Process hello created; pid = 1151

Listening on port 2000

The server for the GNU debugger (gdbserver) is executed to debug the hello program, waiting for a connection on localhost at port 2000.

If the hostname localhost is not recognized, you can instead use the Raspberry Pi’s IP address directly, for example:

root@raspberrypi5:~# gdbserver 192.168.1.2:2000 hello

On the development host PC, run the following command in the directory where the hello executable is located:

~/raspberrypi/user-app$ aarch64-poky-linux-gdb hello

GNU gdb (GDB) 14.2

Copyright (C) 2023 Free Software Foundation, Inc.

License GPLv3+: GNU GPL version 3 or later <http://gnu.org/licenses/gpl.html>

This is free software: you are free to change and redistribute it.

There is NO WARRANTY, to the extent permitted by law.  Type "show copying"

and "show warranty" for details.

This GDB was configured as "--host=x86_64-pokysdk-linux --target=arm-poky-linux".

Type "show configuration" for configuration details.

For bug reporting instructions, please see:

<http://www.gnu.org/software/gdb/bugs/>.

Find the GDB manual and other documentation resources online at:

<http://www.gnu.org/software/gdb/documentation/>.

For help, type "help".

Type "apropos word" to search for commands related to "word"...

Reading symbols from hello...done.

(gdb) target remote 192.168.1.2:2000

Remote debugging using 192.168.1.2:2000

warning: Unable to find dynamic linker breakpoint function.

GDB will be unable to debug shared library initializers

and track explicitly loaded dynamic code.

0x76fcfac0 in ?? ()

(gdb)

This will load the cross GNU debugger that targets the embedded Linux system running on the Raspberry Pi 5 (arm-poky-linux-gdb).

Once the debugger is running, you need to connect to gdbserver to perform a remote debugging session. For this purpose, use the command target remote 192.168.1.2:2000.

You can now debug the program using standard gdb commands. For example, type the following commands:

(gdb) list

1	#include <stdio.h>

2

3	int	main( void )

4	{

5		printf( "Hello, World!\n" );

6

7		return 0;

8	}

9

10

The list command shows you the source code of the program.

You can then set a break point using the break command as follows:

(gdb) break 5

Breakpoint 1 at 0x102f4: file hello.c, line 5.

You can also set the Hardware break point using the hbreak command as follows:

(gdb) hbreak 5

Hardware assisted breakpoint 1 at 0x102f4: file hello.c, line 5.

We are now ready to run the program till the break point is reached:

(gdb) continue

Continuing.

warning: Could not load shared library symbols for 2 libraries, e.g. /lib/libc.so.6.

Use the "info sharedlibrary" command to see the complete listing.

Do you need "set solib-search-path" or "set sysroot"?

Breakpoint 1, main () at hello.c:5

5		printf( "Hello, World!\n" );

To advance the execution step by step, you can use the next command. When the printf() instruction is executed, you will see the “Hello, World!” message appear on the Raspberry Pi 5 terminal.

To terminate the debug session, you can use the quit command.

If your program uses variables, you can inspect them with the print command (refer to the GDB interactive help for syntax details by using the command help print).

## SUMMARY

In this lab, you used Yocto to set up a GCC-based development environment on your host PC, enabling you to cross-compile applications for the embedded Linux system running on the Raspberry Pi 5 and perform remote debugging operations via the host PC terminal.