Quiz Questions – Embedded Linux

Lecture 2:

Question 1 – “The bootloader is a piece of software responsible for…”

- Providing all the services to manage the hardware resources

- Setting up the hardware to run the operating system

- Implementing the functionalities to be delivered to the embedded system user

- Storing the Linux Kernel Configuration files, the system programs, and the application

Question 2 – “In the ‘Reference Hardware Model’, which key component incorporates volatile memory?”

- Boot Flash

- Mass Memory Flash

- RAM

- CPU

Question 3 – “The Linux kernel view of memory is split into…”

- Two layers: User space and kernel space

- Two layers: Developer space and user space

- Three layers: OS, hardware, and software

- Three layers: Networking stack, system stack and application stack

Question 4 – “Which of the following are methods for informing the kernel of which resources are available in the embedded system? (Select all answers that apply)”

- Get the kernel to look it up online

- Use a device tree blob

- Look it up in a memory map

- Hardcode it into the kernel binary code

Question 5 – “What produces a device tree blob (DTB) file?”

- Device Tree Origin

- Device Tree Source

- Device Tree Generation

- Device Tree

Question 6 – “With respect to the bootloader – at power-up, the program counter is set to a default value, known as the…”

- Default vector

- Initial vector

- Primary vector

- Reset vector

Question 7 – “A bootloader operation carried out at the power-up stage is to…”

- Begin executing software instructions from the reset vector

- Preload the boot flash with the bootloader

- Jump to the first instruction of the operating system

- Preload the mass memory flash with the device tree

Question 8 – “What kind of files are typically stored in the /etc directory of the root filesystem?”

- Temporary files

- User data files

- System configuration files

- Binary library files