# introduction to linux kernel modules under yocto

## goal

The goal of this lab is to enable you to build a simple loadable kernel module for Linux on the Raspberry Pi using the Yocto build environment.

## prerequisites

To follow this Lab, you need:

- Raspberry Pi 5 board full;

- USB-C power supply for Raspberry Pi 5 (official 27W Raspberry Pi 5 power supply recommended);

- 8 GB or larger Micro SD card (32 GB card recommended);

- USB-to-Serial debug module for Raspberry Pi 5 or USB-to-TTL adapter;

- A PC with Ubuntu Desktop, or a virtual machine running Ubuntu Desktop with at least 100 GB of free space (recommend Ubuntu 24.04 LTS natively but it’s possible to run via Windows WSL 2);

- A Micro SD card reader attached to the PC/virtual machine;

- (Optional) Micro HDMI cable.

## workplace setup

Assuming you have completed the initial lab, on your Ubuntu host PC, change to the raspberrypi5 directory and prepare the build environment:

cd ~/raspberrypi5/

source sources/poky/oe-init-build-env rpi-build

You now have the system ready to build the embedded Linux distribution for the Raspberry Pi 5 board. We will update the source tree used in the first lab to include a simple device driver.

## Preparing the recipes

In this lab, we are adding a new device driver to be compiled as Linux loadable kernel module, as you would do if you were asked to customize the board support package for the Raspberry Pi 5 to support a new I/O device attached to the system.

For this purpose, we will add a new Yocto recipe to the meta-raspberrypi layer. Create the directory as follows:

cd ~/raspberrypi5/sources/meta-raspberrypi/recipes-kernel

mkdir -p hello-mod/files

In the hello-mod folder, create a file named hello_1.0.bb with the following content:

| 0: 1: 2: 3: 4: 5: 6: 7: 8: 9: 10: | DESCRIPTION = "hello driver" LICENSE = "GPL-2.0-or-later" LIC_FILES_CHKSUM = "file://${BPN}.c;endline=49;md5=4866f9824d27c1cd5324fd5e84caeb6e" inherit module PR = "r0" SRC_URI = "file://Makefile file://${BPN}.c" S = "${WORKDIR}" |
| --- | --- |

This file describes the property of the recipe, and in particular:

- Line 0 gives a simple description of the recipe’s purpose.

- Lines 1-3 reference the license under which the recipe is distributed and provide the indication of the license file as well as its checksum;

- Line 4 indicates that the recipe inherits the properties of the recipes used to build kernel modules;

- Line 6 specifies the machine layer for which the recipe is intended;

- Line 8 lists the source files needed to build the recipe;

- Line 10 sets the symbol pointing to the working directory used to perform the build operations.

In hello-mod/files,create a file named hello.c with the following content:

| 0: 1: 2: 3: 4: 5: 6: 7: 8: 9: 10: 11: 12: 13: 14: 15: 16: 17: 18: 19: 20: 21: 22: 23: 24: 25: 26: 27: 28: 29: 30: 31: 32: 33: 34: 35: 36: 37: 38: 39: 40: 41: 42: 43: 44: 45:46:47: | #include <linux/cdev.h> #include <asm/uaccess.h> #include <linux/module.h> #include <linux/init.h> #include <linux/kernel.h> #include <linux/types.h> #include <linux/kdev_t.h> #include <linux/fs.h> static dev_t hello_dev; struct cdev hello_cdev; static char buffer[64]; ssize_t hello_read(struct file *filp, char __user *buf, size_t count, loff_t *f_pos) { printk(KERN_INFO "Dummy read (count=%d, offset=%d)\n", (int)count, (int)*f_pos ); return 1; } struct file_operations hello_fops = { .owner = THIS_MODULE, .read = hello_read, }; static int __init hello_module_init(void) { printk(KERN_INFO "Loading HelloWorld_module\n"); alloc_chrdev_region(&hello_dev, 0, 1, "hello_dev"); printk(KERN_INFO "%s\n", format_dev_t(buffer, hello_dev)); cdev_init(&hello_cdev, &hello_fops); hello_cdev.owner = THIS_MODULE; cdev_add(&hello_cdev, hello_dev, 1); return 0; } static void __exit hello_module_cleanup(void) { printk(KERN_INFO "Cleaning-up hello_dev.\n"); cdev_del(&hello_cdev); unregister_chrdev_region(hello_dev, 1); } module_init(hello_module_init); module_exit(hello_module_cleanup); MODULE_AUTHOR("Your Name"); MODULE_LICENSE("GPL"); |
| --- | --- |

The file contains the source code of the kernel module;

- Lines 0-7 include the required header files containing the function prototypes and the data structure needed to write a kernel module;

- Lines 9-42 are identical to those discussed during the lecture for module 5;

- Line 44 declares the function used as the module initialization function;

- Line 45 declares the function used as the module cleanup function;

- Lines 46 and 47 declare the author of the source code and the license used to distribute it.

Finally, in the same directory, create a file named Makefile with the following content. Ensure that all indented lines use tabs, not spaces, as using spaces will cause errors later in the lab. You will need to manually add the tabs to the indented lines, as copying and pasting may cause problems.

| 0: 1: 2: 3: 4: 5: 6: 7: 8: 9: 10: 11: 12: 13: 14: | obj-m := hello.o SRC := $(shell pwd) all: $(MAKE) -C $(KERNEL_SRC) M=$(SRC) modules_install: $(MAKE) INSTALL_MOD_DIR=kernel/drivers/my_mod -C $(KERNEL_SRC) M=$(SRC) modules_install clean: rm -f *.o *~ core .depend .*.cmd *.ko *.mod.c rm -f Module.markers Module.symvers modules.order rm -rf .tmp_versions Modules.symvers |
| --- | --- |

The file describes how the source code will be built and where the output binary will be placed, specifically:

- Lines 0-5 define the object module to be created, locate the current directory, and invoke the build command, which refers to the source and kernel source symbols maintained by Yocto;

- Lines 7-9 define the operations required to install the loadable kernel module resulting from the compilation. In this example, the object will be installed in the root file system under the /lib/modules/<kernel version>/kernel/drivers/my_mod directory.

- Lines 11-14 define the operations performed when cleaning the build outputs (that is, removing the temporary files and the compilation outputs).

Once these operations are completed, you have to tell the machine layer configuration to include the new driver. For this purpose, edit the local.conf file

gedit ~/raspberrypi5/rpi-build/conf/local.conf

Add the following statement as the last line of the file:

IMAGE_INSTALL:append = " hello"

Make sure to include a space between the first quotation mark and the start of the module name ‘hello’. This line tells Yocto that when building Linux, the newly created device driver should be built and included in the root file system.

## Building and deploying the new system

You are now ready to build the new system as follows:

cd ～/raspberrypi5/rpi-build

bitbake -c clean rpi-test-image

bitbake rpi-test-image

During the build, the compiler may detect a discrepancy in the license reference. Copy and paste the suggested reference (“the new md5 checksum is..”) into your hello_1.0.bb file where it says "md5=...." and start the build again with

bitbake rpi-test-image

Note: If the `bitbake` execution reports an ERROR, such as a PermissionError or a User namespaces error, you need to make a temporary change to your Ubuntu security profile to enable `bitbake` to run:

sudo su

echo 0 > /proc/sys/kernel/apparmor_restrict_unprivileged_userns

exit

See https://lists.yoctoproject.org/g/yocto/topic/workaround_for_uid_map_error/106192359 for more details

After a while, a new Micro SD card image will be available, which you can deploy to the Micro SD as follows (assuming the Micro SD is available to the PC as /dev/sdb). Alternatively, you can use a program of your choice to flash the image.

First, run the:

sudo fdisk -l

command to determine which device to flash to (plug in and unplug the SD card to identify it). In this example, the SD card is under the name “sdb” (this may differ in your environment). Next, ensure that the device is unmounted. This can be done using the following command:

sudo umount /dev/sdb*

Once this is done, use the following command to copy the image to the SD card (substitute any folder names and device names to match your specific environment).

sudo dd bs=1M if=/home/user/raspberrypi5/rpi-build/tmp-glibc/deploy/images/raspberrypi5/rpi-basic-image-raspberrypi5.rootfs.rpi-sdimg of=/dev/sdb

Note that if this is not done correctly, the image flashed to the SD card may cause problems when attempting to power on the board. If this happens, retry the process and ensure that it is completed properly, or use a flashing program to automate the process.

While the SD card is still connected to the development host, use the following commands to navigate to the etc folder on the SD card (assuming the device has been mounted).

If it has not been mounted, then mount it with the following command:

sudo mount /dev/sdb2 /mnt

cd /media/user/SD_name/etc – Use the ls command to find the name of the SD card in the user folder.

Then, use a terminal text editor to open the shadow file.

sudo vim shadow

or

sudo gedit shadow

Check that there are no characters between the first two colons in the first line. If there are, remove them so that the first line appears as follows:

root::17728:0:99999:7:::

Then, save the file and exit the text editor.

## Running the module

After booting the new Linux system, you can verify whether the build process was completed successfully. After logging into the Raspberry Pi 5 as the root user (no password), enter the following commands:

root@raspberrypi5:/# ls -la /lib/modules/6.x.xx/kernel/drivers/my_mod

drwxr-xr-x  3 root root 1024 May 15 08:47 .

drwxr-xr-x 16 root root 1024 May 10 12:09 ..

-rw-r--r--  1 root root 5384 May 11 08:05 hello.ko.xz

root@raspberrypi5:/#

If you can’t find the directory containing the kernel module, change to the drivers directory and run the following  command:

root@raspberrypi5:/# find . -name "hello.ko.xz"

This directory in the root file system contains hello.ko.xz, which is the compressed kernel object that includes the binary code for the loadable kernel module.

You can now insert the module into the kernel as follows:

root@raspberrypi5:/# insmod /lib/modules/6.x.xx/kernel/drivers/my_mod/hello.ko.xz

[ 363.669107] Loading HelloWorld_module

[ 363.672846] 243:0

The output messages indicate that the module has been loaded correctly, with major number 243 and minor number 0.

If this output is not immediately visible, you can view it using the dmesg command, which displays kernel log information.

You can attach this module to a file with the mknod command:

mknod /dev/hello c 243 0

and then read data from it with cat, head, or dd.

Check the kernel logs again with dmesg to see the read actions taking place.

Finally, to remove the kernel module, do the following:

root@raspberrypi5:/# rmmod hello

[ 475.900838] Cleaning-up hello_dev.

The message indicates that the module cleanup function executed correctly.

# post-lab practice

From this lab session onward, we will implement a bigger challenge using the Raspberry Pi 5 board to accomplish a practical task. The overall workload is evenly distributed across each lab session, and failing to complete this session will affect the next two sessions. In this session, we will continue to practice with kernel modules and BitBake usage.

Configure the corresponding files to autoload the “hello” kernel module.

Assuming you have completed the lab exercise above, remove the SD card from the Raspberry Pi 5 board, insert it into the SD card reader, and re-connect it to your host PC.

The required step for autoloading a module is to modify the local.conf file located in the raspberrypi5/rpi-build/conf/ directory. Open it and add a new line at the end of the file:

KERNEL_MODULE_AUTOLOAD:append = " hello"

The “hello” refers to the name of the kernel module to be autoloaded. If you want to autoload multiple modules, simply list them with a single space between each.

Then, switch to the “~/raspberrypi5” directory and setup the build environment using the following command:

source sources/poky/oe-init-build-env rpi-build

Next, make sure you are at the build directory and build the new system using:

bitbake -c clean rpi-test-image

bitbake rpi-test-image

Copy the new Micro SD card image as done in the labs.

sudo dd bs=1M if=/home/user/raspberrypi5/rpi-build/tmp-glibc/deploy/images/raspberrypi5/rpi-test-image-raspberrypi5.rpi-sdimg of=/dev/sdc

Now test whether the module is autoloaded. Insert the SD card back into the Raspberry Pi 5 board and boot it. Connect your Raspberry Pi 5 board to the host PC and open Minicom, or connect the Raspberry Pi 5 board to an HDMI display and keyboard.

After logging in as the root user (no password), run the `dmesg` command. You should see in the boot logs that the HelloWorld_module was automatically loaded during boot.

Running the following command should return an error because the module is already loaded:

insmod /lib/modules/6.x.xx/kernel/drivers/my_mod/hello.ko.xz

The error reported should say:

ERROR: could not insert module … hello.ko.xz: File exists

Alternatively, you can remove the kernel module immediately after booting by using the following command. This command should execute successfully.

rmmod /lib/modules/6.x.xx /kernel/drivers/my_mod/hello.ko.xz